Acoustical Society of America (ASA) Working Group S3-79

ANSI S3.5-201X: Methods for the calculation of the Speech Intelligibility Index

Extending the SII method for a better prediction of the speech intelligibility in fluctuating noise

Author: Koenraad S. Rhebergen

Department of Otorhinolaryngology and Head & Neck Surgery, University Medical Center Utrecht, Utrecht, The Netherlands

Brain Center Rudolf Magnus, University Medical Center Utrecht, Utrecht, The Netherlands

Email: k.s.rhebergen@umcutrecht.nl

Draft for extension to ANSI standard S3.5-1997; Section 5 after present sections 5.3 version II.

5.4	Extending the SII method for a better prediction of the speech intelligibility in fluctuating noise

5.4.1	Aim: To extend the SII method to calculate the Speech Reception Threshold in fluctuating noise or to describe the speech audibility as a function of SNR in non-stationary noise.

Section 5.4.2 and 5.4.3 describes the basic principle of the extension to the SII model according to Rhebergen and Versfeld (2005) to predict the Speech Reception Threshold (SRT; Plomp and Mimpen, 1979) in stationary and fluctuating noise. Section 5.4.4 describes the extension to the SII model which account for forward masking according to Rhebergen et al (2006).

5.4.2	Introduction

Since the SII calculation scheme (as described in section 4 of ANSI S3.5) uses the long-term averaged speech and noise spectrum as input, all temporal characteristics of these signals are lost. However, large differences in intelligibility exist between masking noises that differ from each other solely with respect to temporal fluctuations (e.g., steady state versus fluctuating noise). In this section, an extension is proposed that does take the temporal characteristics of the masking noise into account. An extension to the conventional SII model of section 4 is described in order to be able to predict the SRT (Plomp and Mimpen, 1979) not only in stationary noise, but also in fluctuating noise. The extension consists of an approach where, for a given condition, both speech and noise signal are partitioned into small time frames. Within each time frame, the conventional SII is determined, yielding the speech information available to the listener at that time frame. Next, the SII values of these time frames are averaged, resulting in the SII for that particular noise type. The results from a series of experiments show that this averaged SII is closely related to the speech intelligibility for that condition.

5.4.3	Description and background of the Extended SII approach

(1)	A block diagram of the calculation scheme is presented in Rhebergen and Versfeld (2005, Fig. 1). Both speech and noise are analyzed separately for the SII calculation. Although, in principle, actual speech could be used as the speech input signal, speech-shaped noise (i.e., stationary gaussian noise with the long-term average spectrum of speech) was used. The main reason for this is that in combination with stationary noise as a masker, all SII values become identical to those obtained with the existing SII model. This prerequisite is not easily fulfilled when actual speech signals would be used.

The SII is in principle designed to predict the average intelligibility of speech in noise and not the intelligibility of individual words or phonemes. In any case, the SII is badly defined in case of silent periods that occur in the normal speech signal because, regardless of the masking noise, the SII will always be zero. Thus, due to the inherent silent periods in the speech signal, even when a speech signal is presented at a clear level without any masking noise, the SII based on regular speech never will reach unity. Moreover, problems will occur if one considers the silent periods between sentences. It is clear that large differences in SII arise when the silent periods between sentences vary, whereas the actual intelligibility should not be different.

(2)	The most straightforward approach to determine the SII within small time frames is to window the speech and noise signal at a given point in time, calculate the frequency spectrum (by means of a Fast Fourier Transform, FFT), and derive an SII from the resulting speech and noise spectrum and the threshold of hearing. However, in order to be able to track the perceptually relevant fluctuations over time, the window length should be small enough. This means that the time window should have a duration of several milliseconds, which is the temporal resolution for normal-hearing listeners based on gap detection thresholds in the higher frequency bands (Shailer and Moore, 1983, 1987; Glasberg and Moore, 1992). Unfortunately, such a short time window leads to the signal-analytical problem that the level in the lower frequency bands is not estimated accurately. On the other hand, a longer time window leads to a poorer representation of the temporal variations of the signal.

(3)	It is known that the temporal resolution of the auditory system is frequency dependent (Moore, 1997). Time constants (i.e., integration times) for the lower frequency bands are larger than those for the higher bands. To overcome the analysis problems on the one hand, and to stay close to the characteristics of the auditory system with respect to spectro-temporal resolution on the other hand, the signal is filtered into 21 critical bands, and the window length is taken to be relatively short in the higher bands and relatively long in the lower bands. Since in the original SII calculations the frequency bands are essentially non-overlapping (after all, the intensity within each filter band was derived from the frequency spectrum), a FIR filter is used to filter the entire speech and noise signal into the separate bands. Within each band, the temporal envelope is determined by means of a Hilbert Transform. At a given time frame, rectangular windows are used with window lengths ranging from 35 ms at the lowest band (150 Hz), to 9.4 ms at the highest band (8000 Hz). These window lengths were taken from Moore (1997, Chapter 4) for gap detection and have been multiplied by 2.5, taken from Moore (1997). Table I gives the gap duration in ms at the  center frequency of each critical band. By means of interpolation the gap length at other frequencies can be determined. The factor 2.5 was chosen to provide a good fit to the present SRT data-set. The windows are aligned such that they end simultaneously. Within each time frame the intensity is determined, and these, together with the absolute threshold for hearing are used as input to calculate the instantaneous SII at that given time frame. Next, the SII values of these time frames are averaged, resulting in the SII for that particular noise type.

Table I.

The  gap length in ms at a characteristic frequency

| Frequency (Hz) | Gap (ms) |
| --- | --- |
| 150 | 13.75 |
| 250 | 8.88 |
| 350 | 7.63 |
| 450 | 6.94 |
| 570 | 6.79 |
| 700 | 6.63 |
| 840 | 6.40 |
| 1000 | 6.00 |
| 1170 | 6.00 |
| 1370 | 6.00 |
| 1600 | 6.00 |
| 1850 | 6.00 |
| 2150 | 5.78 |
| 2500 | 5.25 |
| 2900 | 4.65 |
| 3400 | 4.30 |
| 4000 | 4.00 |
| 4800 | 4.00 |
| 5800 | 3.96 |
| 7000 | 3.90 |
| 8500 | 3.70 |

(4)	The extended SII approach described by Rhebergen and Versfeld (2005) is based on the critical band calculation by means of a 200th order FIR filterbank. For reliable speech intelligibility calculations in steady state noise the choice of the filter type and filter order is not essential, but in fluctuating noise it is. By using other filter order(s) or other filterbank types such a butterworth, chebyshev or gammatone filterbank, it is recommended to check a valid gap correction factor. The factor 2.5 is valid in combination of a 200th order FIR filterbank to predict the SRT in Rhebergen and Versfeld (2005).

5.4.4	Implementation of forward masking in the Extended SII

A large number of studies have shown the possibility of masking a target signal by a preceding masker (so-called Forward Masking), and its relationship between masker level and the time interval between masker and target signal. These studies show a decrease in masking threshold with increased masker-signal delay. The masking threshold returns in about 200 ms to the level of the unmasked target signal threshold (i.e., when no masker is present), regardless of masker level. When plotting the masking thresholds (dB SPL) as a function of masker-target gap on a logarithmic time scale, a linear relationship exists (e.g., Plomp, 1964). Ludvigsen (1985) has modeled this function, which is called in this paper the forward-masking function (FMF). Ludvigsen (1985) figure 4 page 1277 displays the FMF, i.e., the masked threshold as a function of time for two masker levels (high-level masker and low-level masker) and for a normal-hearing and a hearing-impaired subject. As can be seen in figure 4, the FMF is linear between T0 and Tf when time is plotted on a logarithmic axis. Also, the duration of the FMF is always equal, regardless of hearing loss or masker level.

The middle portion of the FMF is a simple linear relationship, and is given by
EFMF(t)=E(T0)−log(tT0)log(TfT0)∗[E(T0)−E(Tf)],

where at time T0, the level of the linear portion E(T0) is equal to the level of the envelope of the masker, and where at time Tf, the linear function intersects with the absolute threshold of hearing E(Tf). The values of the parameters T0 and Tf are 2 ms and 200 ms, respectively. To take forward masking into account, the original envelope E(t) is modified according to: E(t)=maxE(t),EFMF(t). In this manner, sharp onsets in the envelope are followed instantaneously, but sharp offsets make that EFMF(t) takes over, resulting in a gradual decline of the envelope (due to forward masking). The FMF does not account for the phenomenon of backward masking, where a soft signal is masked by a louder signal that follows it. Backward masking is still poorly understood (Moore, 2003), and its effect on speech intelligibility is still unclear and probably not very large.

Note that the FMF is similar in the low and high frequency bands, whereas temporal integration is not. Both phenomena act separately on the signal, and probably are situated in different places in the auditory system. So far, forward masking was not modeled separately in the extended SII (Rhebergen and Versfeld, 2005), but rather it was taken together with temporal integration. Effectively this gave longer integration times, which can explain that a best fit to the data of Rhebergen and Versfeld (2005) was obtained by multiplication of the time constants from Moore (1997) by a factor of 2.5. However, when forward masking is modeled separately from temporal integration, the original integration times (see table I) should be used. These integration times are valid in combination of a 1000th order FIR filterbank to predict the SRT in Rhebergen et al., (2006).

5.4.5	Limitations of the Extended SII approach

As shown by Rhebergen and Versfeld (2005), this SII extension is a clear improvement to describe the speech audibility in fluctuating noise, but there are still some issues left for enhancement.

(1)	The SII extension described in section 5.4.3 and 5.4.4 (i.e. according to Rhebergen and Versfeld, 2005) has only been validated with SRT data at a fixed noise level of about 65 dBA. Contrary to SRT measurements in stationary noise, the SRT in fluctuating noise depends markedly on noise level (Rhebergen et al., 2010, 2014). To account for noise level effects a forward masking function must be implemented in the calculation scheme. Rhebergen et al., (2006) implemented the forward masking function of Ludvigsen (1985, equation 4, page 1277). For more details see Rhebergen et al., (2006) section VC page 3994 or section 5.4.4. in this paper. The factor 2.5 as described in section 5.4.3 should be 1 if the extended SII calculation is conducted with the forward masking function as described in section 5.4.4.

(2)	The extended SII approach cannot account for the phenomenon of “informational masking” (e.g. Rhebergen, Versfeld and Dreschler, 2005, 2008).

(3)	The extended SII approach has only been validated with SRT measurements for sentences in noise.

# 5.4.6	Informative references

Glasberg, B. R., and Moore, B. C. (1992). "Effects of envelope fluctuations on gap detection," Hear.Res. 64, 81-92.

# Ludvigsen, C (1985). “Relations among some psychoacoustic parameters in normal and cochlearly impaired listeners,” J Acoust Soc Am., 78(4):1271-80.

Moore, B. C. (1997). An Introduction to the Psychology of Hearing, fourth edition, Academic Press (London).

Moore, B. C. (2003). An Introduction to the Psychology of Hearing, fifth edition, Academic Press (London).

Plomp, R. (1964). “Rate of decay of auditory sensation,” J. Acoust. Soc. Am. 36, 277-282.

Plomp, R., and Mimpen, A. M. (1979). "Improving the reliability of testing the speech reception threshold for sentences," Audiology 18, 43-52.

Rhebergen, K. S. and Versfeld, N. J. (2005). “A speech intelligibility index-based approach to predict the speech reception threshold for sentences in fluctuating noise for normal-hearing listeners,” J. Acoust. Soc. Am. 117 (4): 2181-2192.

Rhebergen, K. S., Versfeld, N. J., and Dreschler, W. A. (2005).“Release from informational masking by time reversal of native and non-native interfering speech,” J. Acoust. Soc. Am. 118 (3): 1274-1277.

Rhebergen, K. S., Versfeld, N. J., and Dreschler, W.A. (2006). “Extended speech intelligibility index for the prediction of the speech reception threshold in fluctuating noise,” J. Acoust. Soc. Am. 120 (6), 3988-3997.

Rhebergen KS, Versfeld NJ, and Dreschler WA (2008)."Prediction of the intelligibility for speech in real-life background noises for subjects with normal hearing," Ear & Hearing, 29(2), 169-175.

Rhebergen KS, Versfeld NJ, de Laat JAPM and Dreschler WA (2010)."Modelling the speech reception threshold in non-stationary noise in hearing-impaired listeners as en function of level" International journal of audiology: 49 (11): 856-865.

### Rhebergen, K.S. Pool, R.E. and Dreschler, W.A. (2014). " Characterizing the Speech 	Reception 	Threshold in hearing-impaired listeners in relation to masker type 	and masker 	level, " 135 (3): 1491-1503.

Shailer, M. J., and Moore, B. C. (1983). "Gap detection as a function of frequency, bandwidth, and level," J.Acoust.Soc.Am. 74, 467-473.

Shailer, M. J., and Moore, B. C. (1987). "Gap detection and the auditory filter: phase effects using sinusoidal stimuli," J.Acoust.Soc.Am. 81, 1110-1117.